# APPENDIX

List of Symbols, Notation, and Governing Equations

Active Leaf-Spring Vehicle Vertical Comfort (ALVVC) Model

# A.1  List of Symbols

A.1.1  Displacement & Velocity Symbols

| Symbol | Description | Units |
| --- | --- | --- |
| Zs | Bounce of Sprung mass | m |
| Z1 | Bounce of first Unsprung mass | m |
| Z2 | Bounce of second Unsprung mass | m |
| Z3 | Bounce of third Unsprung mass | m |
| Zṡ | Bounce rate of Sprung mass | m/s |
| Ż1 | Bounce rate of first Unsprung mass | m/s |
| Ż2 | Bounce rate of second Unsprung mass | m/s |
| Ż3 | Bounce rate of third Unsprung mass | m/s |
| Z̈s | Acceleration of Sprung mass bounce | m/s² |
| Z̈1 | Acceleration of first Unsprung mass bounce | m/s² |
| Z̈2 | Acceleration of second Unsprung mass bounce | m/s² |
| Z̈3 | Acceleration of third Unsprung mass bounce | m/s² |
| Zrl1 | Road Input on left tire of first axle | m |
| ZrR1 | Road Input on right tire of first axle | m |
| Zrl2 | Road Input on left tire of second axle | m |
| ZrR2 | Road Input on right tire of second axle | m |
| Zrl3 | Road Input on left tire of third axle | m |
| ZrR3 | Road Input on right tire of third axle | m |

A.1.2  Rotational Motion Symbols

| Symbol | Description | Units |
| --- | --- | --- |
| θ | Pitch of Sprung mass | rad |
| θ̇ | Pitch rate of Sprung mass | rad/s |
| θ̈ | Acceleration of Sprung mass pitch | rad/s² |
| ϕs | Roll of Sprung mass | rad |
| ϕ1 | Roll of first Unsprung mass | rad |
| ϕ2 | Roll of second Unsprung mass | rad |
| ϕ3 | Roll of third Unsprung mass | rad |
| ϕ̇s | Roll rate of Sprung mass | rad/s |
| ϕ̇1 | Roll rate of first Unsprung mass | rad/s |
| ϕ̇2 | Roll rate of second Unsprung mass | rad/s |
| ϕ̇3 | Roll rate of third Unsprung mass | rad/s |
| ϕ̈s | Acceleration of Sprung mass roll | rad/s² |
| ϕ̈1 | Acceleration of first Unsprung mass roll | rad/s² |
| ϕ̈2 | Acceleration of second Unsprung mass roll | rad/s² |
| ϕ̈3 | Acceleration of third Unsprung mass roll | rad/s² |

A.1.3  Stiffness & Damping Parameters

| Symbol | Description | Units |
| --- | --- | --- |
| K1 | Combined stiffness for leaf spring of first axle | N/m |
| K2 | Combined stiffness for leaf spring of second axle | N/m |
| K3 | Combined stiffness for leaf spring of third axle | N/m |
| Kr1 | Combined rotational stiffness for leaf spring of first axle | Nm/rad |
| Kr2 | Combined rotational stiffness for leaf spring of second axle | Nm/rad |
| Kr3 | Combined rotational stiffness for leaf spring of third axle | Nm/rad |
| Kt1 | Combined stiffness for tire of first axle | N/m |
| Kt2 | Combined stiffness for tire of second axle | N/m |
| Kt3 | Combined stiffness for tire of third axle | N/m |
| Ktr1 | Combined rotational stiffness for tire of first axle | Nm/rad |
| Ktr2 | Combined rotational stiffness for tire of second axle | Nm/rad |
| Ktr3 | Combined rotational stiffness for tire of third axle | Nm/rad |
| C1 | Combined damping for leaf spring of first axle | Ns/m |
| C2 | Combined damping for leaf spring of second axle | Ns/m |
| C3 | Combined damping for leaf spring of third axle | Ns/m |
| Cr1 | Combined rotational damping for leaf spring of first axle | Nms |
| Cr2 | Combined rotational damping for leaf spring of second axle | Nms |
| Cr3 | Combined rotational damping for leaf spring of third axle | Nms |

A.1.4  Mass & Inertia Parameters

| Symbol | Description | Units |
| --- | --- | --- |
| Ms | Sprung mass | kg |
| M1 | Unsprung mass of first axle | kg |
| M2 | Unsprung mass of second axle | kg |
| M3 | Unsprung mass of third axle | kg |
| Iyy | Pitch moment of inertia | kgm² |
| (Ixx) | Roll moment of inertia of sprung mass for first axle | kgm² |
| (Ixx)1 | Roll moment of inertia of unsprung mass for second axle | kgm² |
| (Ixx)2 | Roll moment of inertia of unsprung mass for third axle | kgm² |
| (Ixx)3 | Roll moment of inertia of unsprung mass | kgm² |
| Ixy | Cross moment of inertia of pitch and roll | kgm² |

A.1.5  Geometric Parameters

| Symbol | Description | Units |
| --- | --- | --- |
| Sf1 | Distance between CG and tire for first axle | m |
| Sf2 | Distance between CG and tire for second axle | m |
| Sf3 | Distance between CG and tire for third axle | m |
| WT1 | Wheel track for first axle | m |
| WT2 | Wheel track for second axle | m |
| WT3 | Wheel track for third axle | m |
| l1CG | Distance between CG and first axle | m |
| l2CG | Distance between CG and second axle | m |
| l3CG | Distance between CG and third axle | m |
| LDL1 | Distance between spring center and its front end for first axle | m |
| LDL2 | Distance between spring center and its front end for second axle | m |
| LDL3 | Distance between spring center and its front end for third axle | m |
| h1 | Distance between Sprung mass CG and roll center | m |
| h2 | Distance between Unsprung mass CG and roll center | m |
| (hus)1 | Distance between first Unsprung mass CG and road | m |
| (hus)2 | Distance between second Unsprung mass CG and road | m |
| (hus)3 | Distance between third Unsprung mass CG and road | m |
| R1 | Reactions on first axle | N |
| R2 | Reactions on second axle | N |
| β1 | Angle between the leaf spring and fixed link for first axle | deg |
| β2 | Angle between the leaf spring and fixed link for second axle | deg |
| β3 | Angle between the leaf spring and fixed link for third axle | deg |
| φ1 | Angle between the bell crank and fixed link for first axle | deg |
| φ2 | Angle between the bell crank and fixed link for second axle | deg |
| φ3 | Angle between the bell crank and fixed link for third axle | deg |
| α | The angle that the leaf spring makes with the horizontal | deg |
| ϕNRS | The roll change due to Non-Reactive suspension | deg |
| ϕroad | The roll change due to road | deg |

# A.2  Governing Equations of Motion

The following equations describe the full multi-body dynamics of the three-axle vehicle model, encompassing cabin and sprung mass motions (bounce, pitch, roll) and unsprung mass interactions via leaf spring suspension.

Equation 1 — Cabin Bounce (zc)

mc·z̈c + (Ccfl + Ccfr + Ccrl + Ccrr)·Żc − (Ccfl + Ccfr + Ccrl + Ccrr)·Żs

+ (Kcfl + Kcfr + Kcrl + Kcrr)·zc − (Kcfl + Kcfr + Kcrl + Kcrr)·zs

− (Ccfl·lcfcg + Ccfr·lcfcg − Ccrl·lcrcg + Ccrr·lcrcg)·ω̇c

− (−Ccfl·lcf + Ccfr·lcf − Ccrl·lcr − Ccrr·lcr)·ω̇s

− (−Ccfl·b − Ccfr·a − Ccrl·b − Ccrr·a)·ε̇c

− (Ccfl·b − Ccfr·a + Ccrl·b − Ccrr·a)·ε̇s

+ [Kcfl, Kcfr, Kcrl, Kcrr] × [lcfcg, lcrcg] terms · ωc and ωs

+ [Kcfl, Kcfr, Kcrl, Kcrr] × [a, b] terms · εc and εs = 0          ...(1)

Equation 2 — Cabin Pitch (ωc)

Iyyc·ω̈c − (Ccfl·lcfcg + Ccfr·lcfcg − Ccrl·lcrcg − Ccrr·lcrcg)·Żc

− (−Ccfl·lcfcg − Ccfr·lcfcg − Ccrl·lcrcg − Ccrr·lcrcg)·Żs

− (−Ccfl·l²cfcg − Ccfr·l²cfcg − Ccrl·l²crcg − Ccrr·l²crcg)·ω̇c

+ (Ccfl·lcfcg·lcf + Ccfr·lcfcg·lcf − Ccrl·lcrcg·lcr − Ccrr·lcrcg·lcr)·ω̇s

+ cross-product stiffness/damping terms for εc and εs

+ (Mcg·hcp)·ωc + geometric stiffness terms = 0          ...(2)

Equation 3 — Cabin Roll (εc)

Ixxc·ε̈c − (−Ccfl·b + Ccfr·a − Ccrl·b + Ccrr·a)·Żc

+ (Ccfl·b − Ccfr·a + Ccrl·b − Ccrr·a)·Żs

+ cross-product [lcfcg, lcrcg] × [a, b] damping terms · ω̇c and ω̇s

− (−Ccfl·b² + Ccfr·a² − Ccrl·b² + Ccrr·a²)·ε̇c

+ (Ccfl·b² − Ccfr·a² + Ccrl·b² − Ccrr·a²)·ε̇s = 0          ...(3)

Equation 4 — Sprung Mass Bounce (zs)

Ms·z̈s + [cabin-sprung coupling terms via Ccfl, Ccfr, Ccrl, Ccrr]

+ Kf(zs − lf·ωs − z1f) + Fdf

+ K2(zs − z2 − ϕL2·LDL2 − ϕR2·LDR2 + l2·ωs) + C2(Żs − Ż2 + l2·ω̇s)

+ K3(zs − z3 − ϕL3·LDL3 − ϕR3·LDR3 + l3·ωs) + C3(Żs − Ż3 + l3·ω̇s) = 0          ...(4)

Equation 5 — Sprung Mass Pitch (ωs)

Isyy·ω̈s + Isyx·ε̈s + [cabin-sprung pitch coupling terms]

− lf[Kf(zs − lf·ωs − z1f) + Fdf]

+ l2[K2(zs − z2 − ϕL2·LDL2 − ϕR2·LDR2 + l2·ωs) + C2(Żs − Ż2 + l2·ω̇s)]

+ l3[K3(zs − z3 − ϕL3·LDL3 − ϕR3·LDR3 + l3·ωs) + C3(Żs − Ż3 + l3·ω̇s)] = 0          ...(5)

Equation 6 — Sprung Mass Roll (εs)

−(Isxx + ms·h²₁)·ε̈s + Isxy·ω̈s = −ms·g·h1·εs

+ ktf(εs − εf) + ctf(ε̇s − ε̇f)

+ kr1(εs − εr1 − εNRS) + cr1(ε̇s − ε̇r1)

+ kr2(εs − εr2 − εNRS) + cr2(ε̇s − ε̇r2)          ...(6)

Equation 7 — Non-Reactive Suspension Roll (εNRS)

where, for i = 2, 3:

εNRS = (ϕLi·LDLi − ϕRi·LDRi) / Stfi          ...(7)

Equations 8 – 9 — Geometric Compatibility Relations

(zs + li2·ω + Stf2·ε) − (LD + Δl)·sin(ϕ2L − ω) = (zus)2L          ...(8)

(zs + li3·ω + Stf3·ε) − (LD + Δl)·sin(ϕ3L − ω) = (zus)3L          ...(9)

These geometric relations constrain the leaf spring angular displacement to the unsprung mass vertical displacement for the second and third axles.

# A.3  References

The symbols, equations, and model formulations presented in this appendix are derived from:

| [1] | Sharma, R. (2023). Active Leaf-Spring Vehicle Vertical Comfort (ALVVC) — Mathematical Model and Simulation. Thesis, 74 pp. |
| --- | --- |
| [2] | ISO 2631-1:1997 — Mechanical Vibration and Shock — Evaluation of Human Exposure to Whole-Body Vibration. |
| [3] | SAE J2807 — Performance Requirements for Determining Tow-Vehicle Gross Combination Weight Rating and Trailer Weight Rating. SAE International. |

End of Appendix